1. What this code does

This script does the following:

- defines the design variables

- defines the bounds of those design variables

- runs the optimizer

- sends each candidate design to your sizing framework

- computes the objective functions

- saves each generation in the folder through the output function

Each generation is saved separately by the output function.

2. Main parts of the code

The script has four important parts:

- design variables and bounds

- optimizer settings

- objective function

- sizing framework

3. Design variables section

- In the code: nvars = 2;
lb = [10, 10];
ub = [30, 15];

Here:

- nvars is the number of design variables

- lb is the lower bound row vector

- ub is the upper bound row vector

In this example:

- x(1) = wing loading

- x(2) = aspect ratio

So the optimizer is allowed to search only within:

- 10 <= x(1) <= 30

- 10 <= x(2) <= 15

Important rule

The size of lb and ub must match nvars. For example, if you have 4 design variables, then:

nvars = 4;
lb = [10, 8, 0.2, 2];
ub = [30, 15, 0.5, 6];

4. Population size

In the code:

populationSize = max(20,  5 * nvars);

This means the total number of designs in each generation is chosen as 5 times the number of design variables.

For example:

- if nvars = 2, then populationSize = 20

- if nvars = 6, then populationSize = 30

You can keep using this rule unless told otherwise.

5. Crossover fraction

In the code:

crossoverFraction = 0.95;

Keep this fixed at 0.95.

Do not change it in this tutorial setup.

6. Optimizer settings

This section creates the solver options:

options = optimoptions('gamultiobj', ...
'PopulationSize', populationSize, ...
'CrossoverFraction', crossoverFraction, ...
'MaxGenerations', 60, ...
'Display', 'iter', ...
'OutputFcn', @outputfunction);

This means:

- population size is set by populationSize

- crossover fraction is set by crossoverFraction

- total generations = 60

- outputfunction saves each generation

Do not modify this.

7. Running the optimizer

This line runs the optimization:

[xPareto, fPareto, exitflag, output, population, scores] = ...
gamultiobj(@objectiveFunction, nvars, [], [], [], [], lb, ub, [], options);

This is the main optimization call. You do not need to change this line unless you later want to add constraints.

function f = objectiveFunction(x)
       Vehicle = dummySizingFramework(x); 
       obj1 = Vehicle.MTOM;
       obj2 = Vehicle.PayloadMassFraction;
       f = [obj1, -obj2];
end

Here, MATLAB passes a design vector x to the objective function. For example, if there are 2 design variables:

x = [WL, AR]

Then the code sends x into the sizing framework:

Vehicle = dummySizingFramework(x);

The sizing framework returns outputs stored in Vehicle. Then you choose which outputs you want as objectives.

In this example:

- obj1 = Vehicle.MTOM

- obj2 = Vehicle.PayloadMassFraction

Finally, those objectives are returned as:

f = [obj1, -obj2];

8. Minimize vs maximize

This is very important. MATLAB’s gamultiobj minimizes everything.

So:

- if your objective is to minimize, keep it as it is

- if your objective is to maximize, put a negative sign

Example

If you want to:

- minimize MTOM

- maximize payload mass fraction

then write:

f = [obj1, -obj2];

because:

- obj1 is minimized directly

- obj2 is maximized by minimizing -obj2

More examples

Minimize both

f = [obj1, obj2];

Minimize first, maximize second

f = [obj1, -obj2];

Maximize both

f = [-obj1, -obj2];

10. Sizing framework section

This part is only a placeholder example:

function Vehicle = dummySizingFramework(x)
    WL = x(1);
    AR = x(2);

    MTOM = 200 ...
               + 4.0*WL ...
               + 180*AR ...
               + 0.08*(WL - 20)^2;

    PayloadFrac = (80 ...
             + 0.6*(WL - 18)^2 ...
             + 20*(AR - 0.30)^2) / MTOM;

    Vehicle.MTOM = MTOM;
    Vehicle.PayloadMassFraction = PayloadFrac;
end

This is where your own framework should go.

What you should do

Replace this dummy function with your own sizing code.

The important structure is:

- extract the design variables from x

- run your sizing calculations

- store the outputs in a structure

- use those outputs inside objectiveFunction

11. Saved generation files

Your output function saves one file for each generation. So in the folder, you will see files like:

- Generation1.mat

- Generation2.mat

- Generation3.mat

- ...

- last generation file

12. What is inside each saved generation file

Each generation file contains a matrix of size:

PopulationSize x (nvars + number of objective functions)

That means:

- each row = one design in that generation

- first nvars columns = design variable values

- remaining columns = objective function values

Column order

The order is:

[x1, x2, ..., xn, obj1, obj2, obj3, ...]

In your current example:

- column 1 = wing loading

- column 2 = aspect ratio

- column 3 = objective 1 = MTOM

- column 4 = objective 2 = negative payload mass fraction

Note that the saved objective is exactly what you returned in f.

So if you wrote:

f = [obj1, -obj2];

then the file stores:

- obj1

- -obj2

not the positive version of obj2.

13. How to open a saved generation file

To inspect a generation file, use:

load('Generation10.mat')

or open it directly in MATLAB.

If the saved variable inside the file is called b, then you can inspect it by:

Example for the current code

In the current code:

- nvars = 2

- number of objectives = 2

- populationSize = 5*nvars = 10

So the saved matrix size in each generation file should be:

10 x 4

because:

- 2 columns for design variables

- 2 columns for objective functions

So each row is:

[WL, AR, MTOM, -PayloadMassFraction]

If you want 3 objective functions

This setup can also return 3 objectives.

Example:

obj1 = Vehicle.MTOM;
obj2 = Vehicle.PayloadMassFraction;
obj3 = Vehicle.CruiseEnergy;

f = [obj1, -obj2, obj3];

Then the saved generation matrix size becomes:

PopulationSize x (nvars + 3)

If nvars = 2, then it becomes:

PopulationSize x 5

Each row would be:

[x1, x2, obj1, obj2, obj3]

Again, the saved values are exactly what you return in f.